Logbook For Microcontroller

Student name: Son Cao

Student number: 570135

Class: ETI2V.IA-2

Week 1: IO & Interrupts

Goal:

- This week, I learned how to control multiple GPIO pins and handle external interrupts on the Raspberry Pi Pico.

Step 2:  Picotool

- picotool load *.uf2 -f means:

  - load = upload the UF2 file to the Pico over USB.

  - -f = “force” overwrite, even if the Pico is busy.

- picotool info can show device info and firmware status.

Step 3: Scanner Light (Mask Functions)

Using GPIO Mask Functions on the Raspberry Pi Pico:

- I learned about GPIO Mask Functions, which are useful for controlling multiple pins at once instead of one at a time with gpio_put(). This makes the code cleaner and faster.

1. gpio_init_mask(mask)

- Initializes all pins specified in the mask at once.

- mask is a bit pattern where each 1 represents a pin to initialize.

- Example: gpio_init_mask((1u << 15) | (1u << 14)); initializes pins 15 and 14.

2. gpio_set_dir_all_bits(mask, direction)

- Sets the direction (input/output) for all pins in the mask.

- direction = true for output, false for input.

3. gpio_put_all(value)

- Sets all pins at once according to the bit pattern in value.

- Example: gpio_put_all(0xFFFF) turns all pins high.

4. gpio_put_masked(mask, value)

- the state of only the pins in the mask. Other pins remain unchanged.

- Example: gpio_put_masked((1u << 15) | (1u << 14), (1u << 15)) turns pin 15 high and pin 14 low, leaving others unchanged.

5. gpio_set_dir_masked(mask, direction)

- Sets the direction of multiple pins at once, only affecting pins in the mask.

Step 4: External Interrupt Example

How the Program Works:

- Pin Initialization:

- The LED is configured as an output pin.

- The button is configured as an input with an internal pull-up resistor (gpio_pull_up()), meaning the pin is normally HIGH and goes LOW when pressed.

- Interrupt Setup:

  - enables an interrupt on the falling edge (button press).

  - When the button is pressed (signal goes from HIGH -> LOW), the interrupt triggers the callback function.

- Interrupt Service Routine (ISR):

- The function gpio_callback() runs automatically when the button is pressed.

- gpio_xor_mask(1 << LED);

  - toggles the LED pin (XOR flips its current state => if on -> off, if off -> on).

- Main Loop:

- The while(true) loop continuously runs tight_loop_contents(), which keeps the system active but idle.

Question: What does the expression: (1<<LED) | (1<< BUTTON) actually mean ?

- This creates a bitmask with bits for both LED and BUTTON set to 1.

- Example: if LED=10 and BUTTON=2:

  - (1<<10) = 0b0000010000000000

  - (1<<2) = 0b0000000000000100

  - The | combines them => both pins are included.

Question: why is there one general gpio callback ?

- Because the RP2040 uses one shared interrupt handler for all GPIOs.
The callback gets the GPIO number that triggered it, so you can handle multiple buttons in one function.

Question: why is it advisable to use the function: tight_loop_contents() ?

- tight_loop_contents() is a hint to the compiler and debugger that your program is idle here.

- It keeps the CPU busy in a power-efficient idle loop, allowing background tasks like USB I/O to continue and avoiding watchdog resets.

  - Allows the debugger (like Picotool) to show that the CPU is in the loop and not hung.

  - Gives the SDK a hook to optimize power use in the future.

  - Keep the program neat instead of just writing an empty while(1); loop

  - => It is advisable because it shows the program is idle, helps debugging, and can save power instead of just spinning in an empty loop.

Step 5: Scanner Light + Interrupts

- BUTTON_START starts scanning on rising edge (button release).

- BUTTON_STOP stops scanning on falling edge (button press).

- volatile bool scanning ensures ISR updates are seen in the main loop.

- Bitmask handling is clean and correct.

- use tight_loop_contents() while idle

Week 2: Timers

Step 1: Timer

Goal:

- Blink one LED at 2 Hz, i.e. on for 0.5 s, off for 0.5 s, using a hardware timer instead of a delay function.

- Also, print the current system time using printf("%lld\n", time_us_64());.

Explanation of How It Works:

- Timer Setup

- add_repeating_timer_ms() creates a repeating callback that runs automatically every 500 ms.

  - Run the function toggle_led_callback() every 500 ms

  - Because the interval is negative, start the timer immediately and align it to system time boundaries.

- Callback Function

- It toggles the LED (on <-> off) using gpio_put(LED, led_state).

- Prints the current timestamp in microseconds via time_us_64().

- Main Loop

- while (1) tight_loop_contents(); keeps the CPU awake and responsive while the hardware timer runs in the background.

Question: What does the negative delay mean, and why is it handy ?

- A negative delay means the timer will fire its callback at a regular time interval aligned to system time instead of waiting for the first delay to pass.

- For example:

  - add_repeating_timer_ms(500, …) waits 500 ms before the first callback.

  - add_repeating_timer_ms(-500, …) calls the first callback immediately, and then repeats every 500 ms.

- This is handy because it:

  - Makes the timer start instantly.

  - Keeps the timing aligned to exact intervals (e.g. every half-second mark of the system clock).

  - Ensures stable, synchronized timing -- ideal for periodic tasks like LED blinking or sampling sensors.

Question: What is the (small) disadvantage of using a positive delay ?

- A positive delay waits one full period before the first callback.

  - The LED won’t blink immediately (it starts after 500 ms).

  - The timer may drift slightly from exact system time boundaries, leading to small timing inaccuracies over long runs.

Step 2: Oscilloscope

Observation:

- When using a negative delay, the LED toggles immediately and stays perfectly aligned to 0.5 s intervals.

- When using a positive delay, there is a small initial delay before blinking starts, and the timing may drift slightly due to scheduling differences.

- On the oscilloscope, the negative-delay signal is consistent and periodic, while the positive-delay signal may show minor offset or jitter.

Week 3: ADC & UART

Step 1: UART communication

Goal:

- Control 4 LEDs through UART.

- The value received (0x0 -> 0xF) determines which LEDs are ON (binary display).
UART uses:

  - 9600 baud

  - no flow control

  - FIFO disabled

  - RX interrupt to set LEDs immediately when data arrives.

How It Works

- UART setup

- Initializes UART0 (pins 0 and 1).

- Flow control disabled (no CTS/RTS).

- FIFO disabled => every received byte instantly triggers the interrupt.

- The RX interrupt calls your handler on_uart_rx() whenever new data arrives.

- Interrupt handler

- Reads each incoming byte.

- Uses ch & 0x0F to keep only the lower 4 bits.

- Updates LEDs via set_leds() accordingly.

- LED update

- Displays the binary pattern of the 4 lowest bits.

- Example: 0x0F (1111) => all LEDs ON, 0x05 (0101) => LED0 & LED2 ON.

- Loopback test

- Connect TX -> RX with a jumper wire.

- In main(), send a test pattern:

LEDs counting in binary 0000 -> 1111.

- Peer-to-peer test

- Connect TX->neighbour’s RX and RX->neighbour’s TX, with common GND.

- Send values to each other and watch LEDs update live.

Question: what is the default parity, databits and stopbits setting? How can you change this?

- The default UART configuration is:

  - Data bits: 8

  - Parity: None

  - Stop bits: 1

- commonly written as 8N1

- How to change these setting:

  - You can change UART frame format using:

    - uart_set_format(UART_ID, data_bits, stop_bits, parity);

  - Example:

    - uart_set_format(UART_ID, 7, 2, UART_PARITY_EVEN);

    - Parity options available:

      - UART_PARITY_NONE

      - UART_PARITY_EVEN

      - UART_PARITY_ODD

Step 2: AD conversion

Goal:

- Read a potentiometer (connected to GPIO 26 / ADC0) and display the MSB 4 bits on 4 LEDs.

How it works:

- As you rotate the pot, the LEDs show a binary pattern of the top 4 bits of the ADC value.

Step 3: ADC & UART

- The ADC reads the potentiometer value every 500 ms.

- The upper 4 bits are sent through UART using uart_putc().

- The RX interrupt on the receiving Pico (or loopback) updates the LEDs.

This allows:

- Real-time analog-to-digital indication over UART.

- Two Picos to exchange live potentiometer data.

Week 4 - 5: SPI & I2C

Goal:

- Control LEDs using the MCP23S17 SPI I/O Expander.

- Output analog voltages using the SPI DAC.

- Read back DAC voltage using the Pi Pico’s ADC.

- Combine both SPI devices to show data on LEDs and output voltage.

- Compare SPI with I2C in theory and practice.

Step 1: SPI I/O Expander (MCP23S17)

How it works

- The MCP23S17 adds 16 extra GPIO pins via SPI.

- Each group of 8 pins (Port A, Port B) is controlled by internal registers.

Registers Used:

| Register | Address | Purpose | Data Written |
| --- | --- | --- | --- |
| IODIRA | 0x00 | Data direction for Port A (1 = input, 0 = output) | 0x00 (all outputs) |
| GPIOB | 0x0A | Controls output state for Port B LEDs | 0–255 (LED binary display) |

Question: Explain in your own words which control, register and data bytes to use, and how and where you found this information.

- In datasheet (8-bit I/O Expander) page 8, I found the SPI addressing structure as below:

- The device op code is set to 0b01000000, since the chip is only going to send write operations to control the LEDs (bit 0 = 0)

- Secondly, as shown in page 9, I used 2 control registers (IODIR and GPIO):

- Page 10 shows me more about how to use IODIR to set the state of the LEDs, as all outputs (register bits are 0x00, or IODIR, data bits are 0x00, for all the LEDs are output)

- Finally, I use GPIO (reg bits 0x09) to control the state of the LED mask, as shown in page 19 of the datasheet:

- There are 8 data bits (or 1 byte), which come after register bits, each of them represents the state of the LED. This is how I send write operation to I/O expander

Question: Why are SPI Chip Select (CS) pins “low-active”?

- SPI uses an active-low Chip Select (CS) line to enable the correct device.

  - When CS = LOW, the selected chip’s SPI input becomes active.

  - When CS = HIGH, the chip ignores the SPI clock and data lines. This prevents multiple devices on the same SPI bus from responding at the same time.

- This design is hardware-safe because:

  - Devices are inactive by default (CS HIGH).

  - Prevents multiple chips from driving the MISO line at once.

  - Saves power (chip stays idle when not selected).

- Example: If you pull both DAC and IO Expander CS LOW at the same time, both devices will respond and data will collide.

Step 2: SPI DAC

How it works:

- The DAC converts a 10-bit digital value (0–1023) into an analog voltage (0–Vref).

Question: What is the input and output range of the DAC chip?

- Digital input: 0-1023 (10-bit digital number)

- Analog output: 0 V -> Vref (usually 0–3.3V if powered at 3.3V)

- Why: The DAC converts digital signals into a smooth voltage for things like sensors, actuators, or LED brightness control.

Question: Besides the normal operation mode, what are the other modes used for? And how ?

- DACs typically have 3 modes (controlled by configuration bits):

  - Normal mode ( DAC output enabled ): Output follows the input value => Generate analog voltage

  - Shutdown mode (Output buffer off, DAC idle): Reduces current, disables output, or connects output via a resistor to GND => Save power

  - Gain mode (Adjust reference scaling (x1 or x2)): Change voltage range

- How: You set special bits in the 16-bit command sent over SPI.

- Why useful: You might want to turn off the DAC temporarily without cutting power or save battery in a low-power device.

Step 3: Combined SPI DAC + IO Expander

Goal:

- To output a voltage using the DAC (Digital-to-Analog Converter) (0 – 1023 => 0 – 3.3V) and simultaneously display the same value in binary on LEDs connected to the IO expander

- The IO Expander lights up LEDs to represent that same number in binary (0 = all off, 255 = all on).

Question: How do you "switch" between using one SPI chip and another ?

- The Chip Select (CS) pin chooses which device listens.

- Each device has its own CS pin.

- Process:

  - Pull DAC CS LOW => send data => pull HIGH

  - Pull IO Expander CS LOW => send data => pull HIGH

- At no point are both CS LOW; this avoids bus collisions.

- You never pull both CS pins low simultaneously -> that’s how you “switch” between devices.

Question: What is strange about the IO Expander’s behavior?

- When switching rapidly between the DAC and IO Expander( if you don’t wait or pulse CS properly ), the LEDs may flicker or lag slightly.

- This happens because SPI mode settings differ:

  - IO Expander => Mode 0 (CPOL=0, CPHA=0)

  - DAC => Mode 1 (CPOL=0, CPHA=1)

- Each mode change reconfigures the SPI controller mid-loop, introducing tiny timing glitches or flickers.

Step 4: I2C vs SPI Comparison

Question: What is the difference between half-duplex and full-duplex? How does this relate to SPI ?

-	Half-duplex: Send OR receive, not both at once (I2C, UART).

-	Full-duplex: Send and receive simultaneously (SPI).

-	SPI: Full-duplex (MOSI sends while MISO receives simultaneously).

- 	I2C: Half-duplex (master sends, then slave responds).

Question: Why does I2C need resistors? And why doesn't SPI need resistors? Explain the difference.

-	I2C uses open-drain pins (outputs): devices can only pull the line LOW, not HIGH.

-	Pull-up resistors are needed to bring the line HIGH when idle.

-	SPI actively drives both HIGH and LOW signals (uses push-pull outputs), so no resistors are needed.

Question: When connecting multiple devices to the "bus" how do I2C and SPI "select" the correct device? Explain the difference.

-	I2C: each device has a unique address (7-bit or 10-bit); the master sends the address before reading/writing.

-	SPI: each device has its own CS pin; pulling it LOW selects that device.

Question: How is decided who the "master" is and who the "slave" in both I2C and SPI? Explain the difference.

-	I2C: the master is the device that generates the clock. Slaves follow.

-	SPI: the master always generates the clock and controls CS; slaves follow commands.

-	Usually, Pico is master, DAC and IO Expander are slaves.

Question: Explain in your own words the difference between a NACK and an ACK in I2C.

-	ACK (Acknowledge): Receiver pulls SDA LOW after each byte => the slave successfully received a byte (data).

-	NACK (Not Acknowledge): SDA left HIGH => the slave did not receive it or can’t accept more data.

-	It’s a way to confirm successful communication.

Question: How is the byte data send in I2C and SPI: MSB or LSB first ? and can this be changed in the Pi Pico?

-	By default, MSB first (most significant bit sent first) for both I2C and SPI.

-	On the Pi Pico, SPI allows changing to LSB first using spi_set_format().

-	I2C is typically fixed as MSB first.

-	Example: spi_set_format(spi0, 8, SPI_CPOL_0, SPI_CPHA_0, SPI_LSB_FIRST);

- 	Value: 0b11001010

-	MSB first => sends 1 first, then 1, then 0,...

-	LSB first => sends 0 first, then 1, then 0,...

WEEK 6-10: FINAL ASSIGNMENT

Question: what is the highest precision?

- For temperature sensors: Resolution (precision): 0.125 °C (11-bit ADC)

- For ambient light sensors: up to 20 bits resolution.

In your logbook you describe your design and how to "control" the chips, i.e. which bytes should go over the line, how you implemented and tested your setup.

- Spi: control LEDs, default pins

- I2C: control temperature and light sensor (LM75B and APDS respectively)

  - SDA: GPIO 4

  - SCL: GPIO 5

  - OS: (temperature interrupt) GPIO 6 (input, pull-up)

  - INT: (light interrupt) GPIO 7 (input, pull-up)

LM75B: Temperature sensor

- I2C address: 0x49

- Control protocol: Send read/write request/command from master to chip through

- SDA line. For example, reading temperature measurements implemented as below:

- Write TEMP register (0x00) then read 2 bytes, combine into 16-bit and shift to the left side 5 bits (11-bit), multiply with resolution (0.125°C per LSB)

Registers:

APDS: Ambient light sensor

- I2C address: 0x52

- Register:

- Control protocol:

- Initialize: set measurement rate, resolution, gain and then turn ALS on to start measurement, using “i2c_write_blocking” with corresponding register address and data value (specified in datasheet)

  - Read measurements: send a write operation of register “ALS_DATA_0” and then send a read operation, reading 3 bytes, and then process it:

Implementation and testing: print the data with current mode into the serial monitor, as well as display I/O expander LEDs accordingly to reading values

Optional Assignment – Threshold Interrupts

Both sensors support interrupt generation:

LM75B:

- OS (Overtemperature Shutdown) pin goes LOW when temperature > Tos ( 24 °C in my case).

- Configure:

  - Write threshold to Tos register.

  - Write hysteresis value to Thyst.

  - OS pin can trigger LED

APDS-9306:

- INT pin triggers when light exceeds or drops below set thresholds.

- Enable interrupt:

  - Set ALS_INT_EN = 1

  - Set upper and lower threshold registers.

- Can use interrupt to blink LED

- Then clear previous interrupt by reading MAIN_STATUS

Test plan:

- Use bright light to trigger APDS interrupt.

- Heat sensor (with finger) to trigger LM75B threshold interrupt.

- Observe LED or serial output confirming interrupt event.

HOW MY CODE WORKS:

Pin Setup

- SDA (GP4) and SCL (GP5) are used for I2C to send and receive data between the Pico and the sensors.

- SPI pins (GP18, 19, 16) and CS (GP17) are used for the LED expander.

- OS (GP6) is the output pin from the temperature sensor that goes LOW when it’s too hot.

- INT (GP7) is the output pin from the light sensor that goes LOW when the light level is too high or too low.

- BUTTON (GP15) is used to switch between showing temperature or light data.

MCP23S08 (SPI I/O Expander)

- This chip controls the LEDs.

- I send 3 bytes through SPI: [control byte][register][data].

- Example:

  - [0x40][0x09][0xFF] turns all LEDs ON.

  - [0x40][0x09][0x00] turns them OFF.

- In the setup, I set all its pins as outputs, so I can send the sensor data to the LEDs easily.

LM75B (Temperature Sensor)

- The LM75B measures temperature in steps of 0.125°C and works automatically after power-up.

- To read it, I write the register address 0x00 and then read two bytes of data.

- These two bytes are combined into an 11-bit value, which is converted into Celsius.

- The function temp_set_thresholds() sets the temperature limit to 24°C.

- If the temperature goes higher, the OS pin goes LOW and triggers a warning.

APDS-9306 (Light Sensor)

- The APDS-9306 measures ambient light with very high accuracy (20-bit).

- I enable it with [0x00][0x02] and set it to high precision mode [0x04][0x04].

- I also set gain = 3x with [0x05][0x01].

- Then I set two light thresholds (low = 100, high = 16000).

- If the light goes outside that range, the sensor sends an interrupt on the INT pin.

- The Pico reads three bytes of data from the sensor, combines them into one 20-bit value, and shows it on the LEDs and serial monitor.

Button and Interrupts

- All interrupts are handled in one function called gpio_callback().

- If I press the button, it changes between showing temperature or light mode.

- If the OS pin goes LOW, it means the temperature is too high => the Pico flashes the LEDs.

- If the INT pin goes LOW, it means the light is too strong => the LEDs also flash.

- The function flash_warning() makes the LEDs blink three times to show the alert.